## 1. Peer-to-Peer Protocols & Service Models (CS5220-C2-M1-L1)

This lecture explains how peer-to-peer processes communicate within layered network architectures. Key ideas include:

- Peer-to-peer communication: Processes at the same layer exchange Service Data Units (SDUs) and Protocol Data Units (PDUs) to provide services to upper layers.

- Service Models:

  - Connection-oriented vs. Connectionless services.

  - Features such as reliability, ordering, flow control, timing, privacy, multiplexing, etc.

- Message handling:

  - Segmentation, blocking, sequencing, error control.

- End-to-End vs Hop-by-Hop control:

  - End-to-end (e.g., TCP) offers scalability.

  - Hop-by-hop (e.g., HDLC) offers fast local recovery.

- Discusses where functions like error control and flow control should be placed.

Overall: Introduces protocol layering, service features, and the tradeoffs in where to implement reliability and flow control.

## 2. Stop-and-Wait ARQ Protocol (CS5220-C2-M1-L2)

This lecture focuses on automatic repeat request (ARQ) and provides a detailed explanation of the Stop-and-Wait protocol.

- ARQ ensures error-free, ordered, and non-duplicated delivery.

- Three ARQ types introduced:
Stop-and-Wait, Go-Back-N, Selective Repeat.

- Stop-and-Wait mechanics:

  - Sender transmits one frame and waits for an ACK.

  - Uses ACKs, NAKs, CRC, and timers.

- Need for sequence numbers: Handles lost frames, lost ACKs, and duplicate transmissions.

- Shows that 1-bit sequence numbering is sufficient (0 and 1).

- Describes the states and actions of the transmitter and receiver.

- Applications: Bisync, XModem, TFTP.

Overall: Explains Stop-and-Wait ARQ reliability, why sequence numbers matter, and how sender/receiver state machines operate.

## 3. TCP Reliable Stream & Flow Control (CS5220-C2-M2-L1)

This lecture describes how TCP ensures reliable, ordered byte-stream communication.

- TCP characteristics:

  - Connection-oriented, reliable, in-order delivery.

  - Uses Selective Repeat ARQ.

- TCP architecture:

  - Sender/receiver buffers.

  - Segment formation and acknowledgement.

- Sequence numbers:

  - Each segment identifies the first byte number.

  - 32-bit numbers to avoid wrap-around issues.

- Three-way handshake for connection setup.

- Flow control:

  - Receiver advertises window size (WA).

  - Sender must not exceed unacknowledged bytes > WA.

- Retransmission Timeout (RTO):

  - Adaptive based on RTT estimates.

  - Includes RTT variance to avoid premature timeouts.

Overall: Shows how TCP achieves reliability using buffering, selective repeat, flow control, and adaptive timeout.